Version 9.6: 06 April 2022

Safe Control of Operations (SCO) Interface Procedure

| Version | Date | Recorded Changes |
| --- | --- | --- |
| 9.5 | 11 October 2011 | Published |
| Draft v9.6 | 06 April 2022 | Minor error in the Recitals for the LDZ/LDZ (Part 3) agreement in v1.1. |
|  |  | - Include document control. - Remove Offtake contact information |

Introduction

This is the SCO Interface Procedure Document referred to in the Offtake Arrangements Document (OAD) Section C 3.0.

The OAD Template Agreements Document is an Offtake Subsidiary Document and accordingly is subject to the further provisions of OAD Section N1.2.

Co-ordination and Control of Work on Sites containing both National Transmission System and Distribution Network Operational Equipment

1. 	INTRODUCTION

This procedure identifies how work will be controlled on sites containing both National Transmission System (NTS) and Distribution Network (DN) operational equipment.  When work is to be undertaken by either party on such sites, it is necessary for communication to be established between both parties to ensure that the work is co-ordinated to enable its safe execution.

## SCOPE

This procedure applies to all installations where NTS and DN assets are contained within the confines of the same site.

- REFERENCE

The 	The Uniform Network Code Offtake Arrangements Document (OAD) is the basis of the procedures that will apply on site. Unless otherwise specified the latest edition of the OAD applies, including all addenda and revisions.

4. 	PRINCIPLES

The following principles shall be applied to the management of work at jointly occupied sites.

Site Controller

- The Site Owner will nominate a Primary Gas Control Centre or Site Controller who will be responsible for monitoring the site activities, recording work activities and ensuring there are no conflicts with other activities on the site or Network. In the event of this the Primary Gas Control Centre or Site Controller would manage conflicts on the site. In addition a review of works at other Offtakes should be carried out to highlight possible conflicts. This may be a second work contact.

- During normal working hours, prior to starting any work that could affect gas flows or telemetered data, the Team leader will contact the Primary Gas Control Centre or Site Controller (as per Table 1) to obtain permission to carry out work.

- For out of working hours then the Team Leader shall contact the Primary Gas Control Centre Responsible for the site (as per Table 1).  The Primary Gas Control centre will then inform the Site Owner’s duty standby manager.

- Where the work has the possibility to affect gas flows, telemetry signals or remote control of the site then, before start of working, the Team Leader shall contact the relevant control centre responsible for monitoring those assets, (ie where NTS assets are affected then the Gas National Control Centre (GNCC) will be contacted and where Distribution assets are affected then the relevant Distribution Control Centre, as defined in Table 1, will be contacted).

Notification of Planned Work

- Annual maintenance plans will be developed and shared by the Site Owner and the Site User as per the OAD.

- The Site User shall give the Site Owner at least five (5) working days’ notice of the intention to carry out any planned work.

- The Site Owner is not required to inform the Site User of any changes to routine planned work as long as the work does not affect gas flow in another user’s network. Major works construction and modifications will be managed in accordance with the OAD.

Urgent Emergency Work

- Emergency and urgent work does not need prior notification before visiting site, however, before work commences the Primary Gas Control Centre that manages the site must be contacted as per the process detailed in Figure 1.  It will then inform the Site Owner’s standby manager, as an immediate callout.

Competency and Permitry Requirements

- The asset owner is responsible for ensuring that the people working on its assets:

  - Understand and comply with the general site safety rules and procedures.

  - Are competent for the work they are required to carry out.

(Note:  The Site Owner must, under the Electricity at Work Regulations, have in place procedures to ensure that anyone working on the site is competent with regard to electrical systems on sites which in most circumstances have shared/common components).

- The asset owner will be responsible for the permitry associated with work on its own land. The Site Owner is responsible for permitry when outside the leased land and for all electrical and instrumentation works on site. Where there is an easement for a pipeline the owner of the pipeline may not be aware of other issues/services on site.

- Where there is a possibility that the work will affect gas flows in more than one Distribution Network then a Non Routine Procedure will be required and approved by the affected Network Operators.

- Where the flow of gas is not affected then no additional permitry will be issued by the Site Owner to the Site User other than the “Permission for Work to Proceed Form” as described in this process. If the site nominated manager decrees a permit is required to safely manage works on site, permitry could be required including controls where work will be undertaken on the owner’s land. This will ensure the Site Controller can manage PPE, equipment and plant as well as environmental issues.

Managing Work Conflicts

- Where there is conflict between two (2) or more activities on a site and agreement cannot be reached on the activity that should be carried out first, then the Team Leaders of the affected activities shall make contact with their Authorising Engineer and Network Controller to seek resolution.

5.	RESPONSIBILITIES FOR CO-ORDINATION OF WORK

Responsibilities for co-ordination of site works are in defined in five (5) groups:

- Work Delivery		A team or an individual assigned to execute site work.

- Authorising Engineer	The person who has assigned and authorised work to take place.

- Primary Gas Control	The Control Centre that has primary control of the site – as nominated by the Site Owner (either Gas National Control Centre (GNCC) or the relevant Distribution Control Centre as defined in Table 1).

- Team Leader		The person in charge and responsible for the work activity on the site. (normally the designated Competent Person for the activity).

- Network Controller 	The person or persons nominated by the Network Owner to be responsible for monitoring and controlling work on the Network to ensure there are no conflicts between activities that will affect the flow of gas.

- Site Control 		Where the site has a designated Permit Office then a Site Controller is appointed to control all activities on the site. (This applies to St Fergus and Bacton Terminals, and Compressor Sites).

- Site Controller		The person designated to control the activities on site.

6.	EXECUTION OF WORK

Before commencing work which could affect the gas supplies or telemetry at site, Work Delivery must:

- Contact the relevant Gas Control Centre(s) and obtain permission to carry out the work;

- Check the Site Logbook to determine whether any other work is taking place;

- When other work is identified, then:

  - Contact the senior person in charge of the other work;

  - Discuss whether both activities can be executed simultaneously in a safe manner;

  - If decided that simultaneous works are unsafe, then contact their Authorising Engineer to determine which work should be executed first;

  - If necessary the Authorising Engineers from both organisations may need to agree which work should take precedence;

  - If both parties agree that simultaneous works can be executed safely, then jointly complete a “Permission for Work to Proceed Form” and place in the Site Logbook.

Note:	For work on manned NTS Terminals and Compressors a

“Permission for Work to Proceed Form” must be completed.

- Contact the Gas Control Centre for the site to obtain authorisation to proceed with the work;

- Enter details of the work in the Site Owner’s Telemetry Room Logbook;

- At the end of the day Work Delivery must make the site safe and remove all waste materials;

- Contact the Gas Control Centre for the site to obtain permission to leave the site;

- Complete the Site Logbook; and

- Ensure that the site is secure before leaving the site.

7.	RETENTION OF RECORDS

The Site Owner is responsible for providing copies of the “Permission for Work to Proceed Form” on the site so that they are available for use when required.

The Team Leader is responsible for ensuring the “Permission for Work to Proceed Form” (see Appendix A) is completed when required and a copy retained in the Site Owner’s Telemetry Room.

The Site Owner will retain copies of the completed “Permission for Work to Proceed Forms” on site for a minimum of two (2) years after the work is completed.

Table 1 – DN/NTS Offtake – Site Roles and Contact Details

| DN/NTS Offtake Site | Site Type | Site Owner | Primary Gas Control Centre / Site Control | Contact During working hours | Contact OUT of working hours |
| --- | --- | --- | --- | --- | --- |
| ABERDEEN | Compressor | National Grid - NTS | NTS Site Control / GNCC | 01224 748089 | 0870 1910633 |
| ABERDEEN | Offtake | Scotia Gas Networks - DN | SGN Gas Control | 0845 0737957 | 0845 0737957 |
| ALREWAS EM | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| ALREWAS WM | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| ARMADALE | Offtake | Scotia Gas Networks - DN | SGN Gas Control | 0845 0737957 | 0845 0737957 |
| ASPLEY | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| ASSELBY | Offtake | Northern Gas Networks - DN | NGN System Control | 0845 6003171 | 0845 6003171 |
| AUDLEY | Multijunction | National Grid - NTS | GNCC | 0870 1910633 | 0870 1910633 |
| AUDLEY | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| AUSTREY | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| AYLESBEARE | Offtake | Wales & West Utilities - DN | WWU Control Centre | 03301 000061 | 03301 000061 |
| BACTON | Terminal | National Grid - NTS | NTS Site Control / GNCC | 01263 720655 | 01263 720655 |
| BACTON | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| BALDERSBY | Offtake | Northern Gas Networks – DN | NGN System Control | 0845 6003171 | 0845 6003171 |
| BALGRAY | Offtake | Scotia Gas Networks - DN | SGN Gas Control | 0845 0737957 | 0845 0737957 |
| BARRINGTON | Multijunction | Wales & West Utilities - DN | WWU Control Centre | 03301 000061 | 03301 000061 |
| BATHGATE/AVONBRIDGE | Compressor | National Grid - NTS | NTS Site Control/GNCC | 0870 1910633 | 0870 1910633 |
| BATHGATE | Offtake | Scotia Gas Networks - DN | SGN Gas Control | 0845 0737957 | 0845 0737957 |
| BISHOP AUCKLAND | Compressor | National Grid - NTS | NTS Site Control / GNCC | 01388 720651 | 0870 1910633 |
| BISHOP AUCKLAND | Offtake | Northern Gas Networks - DN | NGN System Control | 0845 6003171 | 0845 6003171 |
| BLABY | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| BLACKROD | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| BLYBOROUGH | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| BRAISHFIELD | Offtake | Scotia Gas Networks - DN | SGN Gas Control | 0845 0737955 | 0845 0737955 |
| BRISLEY | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| BROXBURN | Offtake | Scotia Gas Networks - DN | SGN Gas Control | 0845 0737957 | 0845 0737957 |
| BURLEY BANK | Offtake | Northern Gas Networks – DN | NGN System Control | 0845 6003171 | 0845 6003171 |
| CALDECOTT | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| CARESTON | Offtake | Scotia Gas Networks- DN | SGN Gas Control | 0845 0737957 | 0845 0737957 |
| CHOAKFORD | Offtake | Wales & West Utilities - DN | WWU Control Centre | 03301 000061 | 03301 000061 |
| COFFINSWELL | Offtake | Wales & West Utilities - DN | WWU Control Centre | 03301 000061 | 03301 000061 |
| CIRENCESTER | Offtake | Wales & West Utilities - DN | WWU Control Centre | 03301 000061 | 03301 000061 |
| COLDSTREAM | Offtake | Northern Gas Networks - DN | NGN System Control | 0845 6003171 | 0845 6003171 |
| CORBRIDGE | Offtake | Northern Gas Networks - DN | NGN System Control | 0845 6003171 | 0845 6003171 |
| COWPEN BEWLEY | Offtake | Northern Gas Networks - DN | NGN System Control | 0845 6003171 | 0845 6003171 |
| DOWLAIS | Offtake | Wales & West Utilities - DN | WWU Control Centre | 03301 000061 | 03301 000061 |
| DROINTON OFFTAKE | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| DRUM | Offtake | Scotia Gas Networks - DN | SGN Gas Control | 0845 0737957 | 0845 0737957 |
| DYFFRYN CLYDACH | Offtake | Wales & West Utilities - DN | WWU Control Centre | 03301 000061 | 03301 000061 |
| EASTON GREY | Offtake | Wales & West Utilities - DN | WWU Control Centre | 03301 000061 | 03301 000061 |
| ECCLESTON | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| ELTON | Offtake | Northern Gas Networks - DN | NGN System Control | 0845 6003171 | 0845 6003171 |
| EVESHAM | Offtake | Wales & West Utilities - DN | WWU Control Centre | 03301 000061 | 03301 000061 |
| EYE | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| FARNINGHAM | Offtake | Scotia Gas Networks - DN | SGN Gas Control | 0845 0737956 | 0845 0737956 |
| FIDDINGTON | Offtake | Wales & West Utilities - DN | WWU Control Centre | 03301 000061 | 03301 000061 |
| FISHACRE | Block Valve | Wales & West Utilities - DN | WWU Control Centre | 03301 000061 | 03301 000061 |
| GANSTEAD | Offtake | Northern Gas Networks - DN | NGN System Control | 0845 6003171 | 0845 6003171 |
| GILWERN | Offtake | Wales & West Utilities - DN | WWU Control Centre | 03301 000061 | 03301 000061 |
| GLENMAVIS | LNG | National Grid - NTS | NTS Site Control / GNCC | 01236 746400 | 01236 746400 |
| GLENMAVIS | Offtake | Scotia Gas Networks - DN | SGN Gas Control | 0845 0737957 | 0845 0737957 |
| GOSBERTON | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| GREAT WILBRAHAM | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| GUYZANCE | Offtake | Northern Gas Networks - DN | NGN System Control | 0845 6003171 | 0845 6003171 |
| HARDWICK | Offtake | Scotia Gas Networks - DN | SGN Gas Control | 0845 0737955 | 0845 0737955 |
| HOLMES CHAPEL | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| HORNDON | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| HUMBLETON | Offtake | Northern Gas Networks - DN | NGN System Control | 0845 6003171 | 0845 6003171 |
| HUME | Offtake | Scotia Gas Networks - DN | SGN Gas Control | 0845 0737957 | 0845 0737957 |
| ILCHESTER | Offtake | Wales & West Utilities - DN | WWU Control Centre | 03301 000061 | 03301 000061 |
| IPSDEN | Offtake | Scotia Gas Networks - DN | SGN Gas Control | 0845 0737955 | 0845 0737955 |
| KELD | Offtake | Northern Gas Networks - DN | NGN System Control | 0845 6003171 | 0845 6003171 |
| KENN (NORTH) | Offtake | Wales & West Utilities - DN | WWU Control Centre | 03301 000061 | 03301 000061 |
| KINKNOCKIE | Offtake | Scotia Gas Networks - DN | SGN Gas Control | 0845 0737957 | 0845 0737957 |
| KIRKSTEAD | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| LANGHOLM | Offtake | Scotia Gas Networks - DN | SGN Gas Control | 0845 0737957 | 0845 0737957 |
| LEAMINGTON | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| LITTLE BURDON | Offtake | Northern Gas Networks - DN | NGN System Control | 0845 6003171 | 0845 6003171 |
| LITTLETON DREW | Offtake | Wales & West Utilities - DN | WWU Control Centre | 03301 000061 | 03301 000061 |
| LOCKERBIE | Offtake | Scotia Gas Networks - DN | SGN Gas Control | 0845 737957 | 0845 0737957 |
| LOWER QUINTON | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| LUPTON | Multijunction | National Grid - NTS | GNCC | 0870 1910633 | 0870 1910633 |
| LUPTON | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| LUXBOROUGH LANE | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| MAELOR | Offtake | Wales & West Utilities - DN | WWU Control Centre | 03301 000061 | 03301 000061 |
| MALPAS | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| MAPPOWDER | Offtake | Scotia Gas Networks- DN | SGN Gas Control | 0845 737955 | 0845 0737955 |
| MARKET HARBOROUGH | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| MATCHING GREEN | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| MELKINTHORPE | Offtake | Northern Gas Networks - DN | NGN System Control | 0845 6003171 | 0845 6003171 |
| MICKLE TRAFFORD | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| MILWICH | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| NETHER HOWCLEUGH | Offtake | Scotia Gas Networks - DN | SGN Gas Control | 0845 0737957 | 0845 0737957 |
| PANNAL | Offtake | Northern Gas Networks - DN | NGN System Control | 0845 6003171 | 0845 6003171 |
| PARTINGTON | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| PAULL | Multijunction / Offtake | National Grid - NTS | GNCC | 0870 1910633 | 0870 1910633 |
| PETERS GREEN | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| PICKERING | Offtake | Northern Gas Networks - DN | NGN System Control | 0845 6003171 | 0845 6003171 |
| PITCAIRNGREEN OT | Offtake | Scotia Gas Networks - DN | SGN Gas Control | 0845 0737957 | 0845 0737957 |
| PUCKLECHURCH | Offtake | Wales & West Utilities - DN | DNCC | 0331 000061 | 0331 000061 |
| RAWCLIFFE | Offtake | Northern Gas Networks - DN | NGN System Control | 0845 6003171 | 0845 6003171 |
| ROSS | Offtake | Wales & West Utilities - DN | WWU Control Centre | 03301 000061 | 03301 000061 |
| ROSS | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| ROUDHAM HEATH | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| ROYSTON | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| RUGBY | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| SALTEND BP | Offtake | Northern Gas Networks - DN | NGN System Control | 0845 6003171 | 0845 6003171 |
| SALTWICK | Offtake | Northern Gas Networks - DN | NGN System Control | 0845 6003171 | 0845 6003171 |
| SAMLESBURY | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| SEABANK | Offtake | Wales & West Utilities - DN | WWU Control Centre | 03301 000061 | 03301 000061 |
| SHORNE | Offtake | Scotia Gas Networks - DN | SGN Gas Control | 0845 0737956 | 0845 0737956 |
| SHUSTOKE | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| SILK WILLOUGHBY | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| SOUTRA | Offtake | Scotia Gas Networks - DN | SGN Gas Control | 0845 0737957 | 0845 0737957 |
| ST. FERGUS | Terminal | National Grid - NTS | NTS Site Control / GNCC | 01779 838611 | 01779 838611 |
| ST.FERGUS | Offtake | Scotia Gas Networks - DN | SGN Gas Control | 0845 0737957 | 0845 0737957 |
| STEPPINGLEY | Multijunction | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| STRATFORD-UPON-AVON | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| SUTTON BRIDGE | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| TATSFIELD | Offtake | Scotia Gas Networks - DN | SGN Gas Control | 0845 0737956 | 0845 0737956 |
| THORNTON CURTIS | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| THRINTOFT | Offtake | Northern Gas Networks - DN | NGN System Control | 0845 6003171 | 0845 6003171 |
| TOW LAW | Offtake | Northern Gas Networks - DN | NGN System Control | 0845 6003171 | 0845 6003171 |
| TOWTON | Offtake | Northern Gas Networks - DN | NGN System Control | 0845 6003171 | 0845 6003171 |
| TUR LANGTON | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| WALESBY | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| WARBURTON | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| WEST WINCH | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| WESTON POINT | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| WETHERAL | Offtake | Northern Gas Networks - DN | NGN System Control | 0845 6003171 | 0845 6003171 |
| WHITWELL | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| WINKFIELD SE | Offtake | Scotia Gas Networks - DN | SGN Gas Control | 0845 0737956 | 0845 0737956 |
| WINKFIELD SO | Offtake | Scotia Gas Networks - DN | SGN Gas Control | 0845 0737955 | 0845 0737955 |
| WINKFIELD NT | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |
| YELVERTON | Offtake | National Grid - DN | DNCC | 0870 2418701 | 0870 2418701 |

Figure 1 – Co-ordination and Control of Site Work

APPENDIX A	“Permission for Work to Proceed Form”

| ## Site Name |  |  |  |  |
| --- | --- | --- | --- | --- |
| ### Is there any other work being carried out on site? - YES / NO |  |  |  |  |
| ### For example, are the following work types being undertaken? (Note: This is not an exhaustive list) |  |  |  |  |
| - Gas being vented will affect work areas - Valve operations will affect gas flows - Electrical isolation will affect valve movements |  |  | - Plant/vehicle movement will conflict with each other - Emergency Access/Egress is affected - Emergency action will affect other work activities |  |
| If YES: Make contact with the Team Leader carrying out work and determine if there is any conflict between their work and your planned work ## If NO: Carry out works as per company instructions and contact relevant bodies. |  |  |  |  |
| ACTIVITY – 1 Validity Period from………….hrs on………….to…………..hrs on……………. |  |  |  |  |
| ### Location of plant/work area: | ### Details of work being carried out: |  |  |  |
| ACTIVITY – 2 Validity Period from………….hrs on………….to…………..hrs on……………. |  |  |  |  |
| ### Location of plant/work area: | ### Details of work being carried out: |  |  |  |
| If there is conflict and the work cannot be carried out safely and agreement cannot be reached on which activity should proceed then contact AUTHORISING ENGINEER for advice. |  |  |  |  |
| There is no conflict between the work and both activities can be carried out safely at the same time. Team Leader in charge of Activity 1 |  |  |  |  |
| Print Name: |  | Signature: |  | Date: |
| There is no conflict between the work and both activities can be carried out safely at the same time. Team Leader in charge of Activity 2 |  |  |  |  |
| Print Name: |  | Signature: |  | Date: |

| Work Complete |  |  |
| --- | --- | --- |
| Print Name: | Signature: | Date: |

NOTES:

- Where more than two activities are being carried out on one site append an additional form and change activity numbers on appended form, e.g. change Activity 1 to Activity 3, etc.

- A copy of this “Permission for Work to Proceed Form” must be retained in the Site Owner’s Telemetry Room.